Zhenlong Township Geospatial Data Description

This data package provides a 30 m Digital Elevation Model (DEM), roads, settlement points, water bodies (polygons), waterways (lines), and an interactive HTML map for Zhenlong Township and the surrounding area. The DEM is provided in both GeoTIFF and MAT formats in the corresponding folder. Roads, settlement points, water bodies (polygons), and waterways (lines) are each provided in both CSV and MAT formats in their respective folders. For each data category, the two formats contain identical data, allowing the data to be read in different software environments.

1. Available Files and Data Contents

| Data Category | Directory or File |
| --- | --- |
| Digital Elevation Model | Digital Elevation Model (DEM) Data/30 m DEM for Zhenlong Township and Surrounding Areas.tif, .mat |
| Roads | Roads/Roads in and around Zhenlong Township.csv, .mat |
| Settlement Points | Settlement Points/Settlement Points in and around Zhenlong Township.csv, .mat |
| Water Bodies (Polygons) | Water Bodies (Polygons)/Water Bodies in and around Zhenlong Township.csv, .mat |
| Waterways (Lines) | Waterways (Lines)/Waterways in and around Zhenlong Township.csv, .mat |
| Interactive Map | Detailed Geospatial Map of Zhenlong Township.html |

2. Coordinate and Elevation Conventions

The DEM is derived from Copernicus DEM GLO-30 and is a Digital Surface Model (DSM). The coordinate reference system is WGS 84 (EPSG:4326), with a pixel size of 1/3600° in both the longitude and latitude directions and a nominal ground resolution of 30 m. Raster center coordinates range from 109.0328°E to 109.4453°E and from 22.8614°N to 23.2247°N. Valid elevations range approximately from 41.7 m to 1132.9 m, and the NoData value is −32767. The raster values, coordinates, and NoData conventions are consistent between the GeoTIFF and MAT files.